MN Antenna Analysis

MN is an enhanced version of the MININEC antenna analysis program for

IBM-PC and compatible computers.  MN is much easier to use than MININEC, and is

oriented toward amateur radio.  No changes have been made to the antenna

modeling algorithm contained in MININEC Version 3 Change 11, as released by the

Naval Ocean Systems Center (except to fix a bug involving Laplace Transform

loads), but substantial changes have been made in the way the program interacts

with the user, and many new features have been added.

This note is just intended to help you with aspects of the program that

are not readily apparent.  How to run the program is largely self-explanatory,

and can be learned by experimenting with the antenna files provided.

New Features

The most important new feature is a provision for antenna files.  This

allows the antenna characteristics to be captured from a text file instead of

from the keyboard, saving you the considerable trouble of reentering a

complicated antenna geometry by hand every time you start up the program.  It

permits you to interrupt your work, save it away, and come back to it later

when it is convenient, without having to start from scratch.  It also enables

you to build up your own library of designs, from which you can easily retrieve

and modify antennas for new applications.  You can use any text editor or word

processor to create the antenna file, as long as it comes out in ordinary ASCII

characters.  Commands are available for listing, loading, viewing, and editing

antenna files.

If you have a graphics display card (CGA, EGA, or HGC) you can plot the

far-field azimuth and elevation patterns directly on your screen.  You may

create polar plots using the standard ARRL log dB plotting scale.  These

patterns may be directly compared with those in amateur antenna publications.

You may also plot in a linear dB scale so that small sidelobes are easier to

see.  Alternatively, the rectangular plot reveals all pattern detail uniformly.

The plots are saved automatically and can be viewed at a later time without

redoing the analysis.  For the EGA and HGC a comparison mode allows you to

switch screens instantly between the plots of two antennas without having to

wait while the plots are redrawn.  This provides a very revealing pattern

comparison.  You can make hardcopies of the screen plots on printers having

Epson-compatible dot graphics.

A new command has been added to compute forward gain, front-to-back

ratio, maximum sidelobe level, beamwidth, and vertical angle of radiation.  You

no longer have to extract these antenna parameters manually by examining

tabulated directive pattern listings.

The SWR is computed and displayed in two ways, revealing both the match

to a given feedline and the inherent bandwidth properties of the antenna.

The maximum number of pulses has been increased from 50 to 126.  This

allows complex antenna systems to be modeled, including complete multiband

antenna installations in order to find interactions among individual antennas.

You may specify the antenna dimensions in feet, inches, meters,

centimeters, or millimeters.  Wire diameter is used instead of radius, and you

may define this by specifying the wire gauge.  You may choose to have results

displayed in dBd or dBi.  Elevation angle (the angle with respect to the

horizon) is used instead of zenith angle (the angle with respect to overhead).

A single file containing a record of the data generated during the antenna

analysis is automatically created and given the same name as the antenna file,

with the extension .RUN.

The all-capital-letters display style has been done away with, and

upper or lower case is accepted from the keyboard.  Defaults are provided for

all keyboard inputs and are indicated by [brackets].  Entering just a carriage

return selects the default value.

System Requirements

MN.EXE requires an IBM-PC or compatible computer with at least 300K

bytes free memory.  You need at least 450K bytes if you invoke PLOT.EXE

directly from MN.EXE, but only 150K bytes when PLOT.EXE is executed by itself.

A Color Graphics Adapter, Enhanced Graphics Adapter, or Hercules Graphics Card

is required for plotting.  Both programs will run much faster if a math

coprocessor chip is installed, but one is not required.  A hard disk is not

necessary.  An editor or wordprocessor program is needed to create or modify

antenna files.  DOS 3.00 or later is recommended to avoid certain bugs in

earlier DOS versions which may impact MN.

Antenna Files

All characteristics of an antenna system, except for details concerning

the ground environment, are specified in an antenna file.  An antenna file must

have the extension ANT.  The antenna file can be specified as a command line

parameter when starting MN; otherwise you will be prompted for it.  Similarly,

when changing to another antenna using the A command, you may specify the new

filename as a command line parameter; otherwise you will be prompted for it.

In any case you need not enter the ANT extension, it will be supplied

automatically.  You may specify a drive and path ahead of the filename if

necessary.

The format of an antenna file is illustrated below by a sample file for

a 3 element beam.  In actual antenna files no comments are permitted on the

lines containing antenna information, but they may be added freely at the end.

3 Element Yagi	  {1 line title for the antenna}

Free Space	  {Use any other character string for antennas over ground}

14.2 MHz	  {"Hz", "KHz", or "GHz" may also be used}

3 wires, feet     {Also "inches", "meters", "centimeters", or "millimeters"}

10  -8,-17,0    -8,17,0   .125	 {For each wire: # segments, XYZ coordinates

10  0,-16.5,0   0,16.5,0  1.5"			  of each end, and diameter}

10  7,-16,0     7,16,0    1.5in  {All 3 wires are 1.5 inches in diam}

1 source	  {Number of sources}

14,100,0	  {Pulse number, voltage, phase of the first source}

0 loads		  {Number of loads}

Segments, sources, pulses, and loads will be explained in subsequent

sections.

To increase readability any combination of spaces, commas, or tabs may

be used as item separators.  Any combination of upper and lower case is valid.

The words "wires", "source", and "loads" may be singular or plural.

To use a different dimension unit for an individual length you may

suffix the number with one of the following:

ft  '  in  "  m  cm  mm

Don't put a space between the numerical value and the unit abbreviation.

You may also specify the wire diameter by entering the wire gauge, for

example, #12.  A wire table is built into the program for even-numbered

standard annealed bare copper wire gauges from AWG 0 through 30.  Odd gauge

numbers are converted to the next larger wire diameter.

Coordinate System

MN uses the Cartesian coordinate system.  X and Y are in the horizontal

plane and Z is height.  In MN, if +X is North then +Y is West.  MN maintains

the original MININEC azimuth angle convention, which is counterclockwise and

reversed from normal compass bearings:  0 degrees azimuth angle is along the +X

direction, and 90 degrees azimuth is along the +Y direction.  +Z is up.  The

forward horizon is at 0 degrees elevation angle, 90 degrees is overhead, and

180 degrees is the rear horizon.  The 180 to 360 degree positive angles can

also be referenced by -180 to 0 degree negative angles.  Unidirectional

antennas should be aimed in the +X direction so that forward gain, F/B, and

beamwidth may be computed automatically.  The Z coordinate can be set to 0 if

only free space modeling is performed.

To gain a mental picture of the geometry involved, imagine facing a 3

element beam 5 feet off the ground which is aimed at you.  Assume that the

center of the driven element has been put at X=0, Y=0, Z=5.  The main lobe of

the beam points directly at you, in the +X direction.  The elements extend in

the -Y direction to your left, and in the +Y direction to your right.  The

director has a +X coordinate, while the reflector is in -X territory.  Positive

azimuth angle is to your right (counterclockwise when viewed from above), and

positive elevation angle is above your head.

Wires, Segments, and Pulses

A wire is always straight in MN.  A bent wire is modeled by connecting

two straight wires.  Two or more wires are considered connected when they share

the same XYZ coordinates at an endpoint, and current will be allowed to flow

between the wires as if they were soldered together.  For example, each loop of

a cubical quad antenna is described by four separate wires whose endpoints lie

at four points.  A 2 element quad is thus modeled by 8 wires in MN, even though

a real antenna would actually have only 2 wires strung around the spreaders.  A

Yagi element composed of tapered sections of telescoping tubing may be

accurately modeled by using several connected wires having different diameters.

Wires which cross or which terminate at midpoints of other wires are not

considered to be connected by MN.  Connections are allowed only at wire ends.

MN allows you to specify how many segments each wire is divided into

for analysis purposes.  Generally, the more segments used the higher the

accuracy, but the longer the analysis takes.  The number of segments required

depends on the geometry of the antenna being analyzed and the accuracy

required.  For example, 5-25 segments are generally used for dipole elements

and 15-100 for full-wave loop elements.  Many segments are required for

antennas having very closely-spaced wires, such as folded dipoles.  To be sure

that you are using enough segments you should always increase the number of

segments and try a new run to see if the results change significantly.  This is

very important.

MN uses the segmentation to divide the current in the wire into

sections called pulses.  The current is uniform within each pulse, and the

pulses are centered on segment boundaries.  No pulses are placed at wire ends,

where the current is always zero, but a pulse is placed at every wire junction,

and overlaps onto all wires making up the junction.  The number of segments

(not pulses) for each wire is specified in the antenna file.  MN will allocate

the pulses after unscrambling all the wire connections.  The number of wires is

limited to 50, the number of pulses to 126, and the number of segments to 176.

Sources

A source of energy (feedpoint) is permitted at any pulse.  You must

have the feed symmetry in mind when you divide the driven wires into segments.

In the Yagi example above, the driven element uses an even number of segments

(10).  This results in an odd number of pulses between the segments (9), and

thus a centrally located pulse at the feedpoint (pulse #14, including the 9

pulses in the reflector wire which was specified first).  Specifying an odd

number of segments for the driven element would result in an off-center

feedpoint.  The first time MN is run with a new antenna file, the .RUN output

file generated by MN should be examined to verify that the pulses have been

distributed and numbered the way you intended.  In fact, the easiest way to

figure out which pulse number to specify as the feed point is by making an

initial guess, loading the antenna file, quitting, and then examining the

antenna geometry section of the .RUN file to see how the pulses really were

allocated.

If an antenna has only one source, and you are not interested in the

magnitude of the antenna currents, then the voltage can be any nonzero value

and the phase can be set to 0.  For two or more sources the relative magnitudes

and phases of the sources will directly influence the antenna characteristics.

Loads

You may specify up to 50 lumped loads for an antenna.  Typical amateur

antennas using lumped loads are triband beams, trap dipoles and verticals, and

loaded dipoles and whips.  Loads are always located at pulses.  There are two

different load models, impedance loads and Laplace Transform loads.  An antenna

may not mix the two kinds.  Here is an example of an antenna using one

impedance load, where the load is a simple resistor:

ZL1ACW's Big Rhombic

35 feet up    {Since not "free space" this antenna will be modeled over ground}

24.94 MHz

4 wires, feet

25   -182.7 0 35      0 81.35 35   #12	 {Wire gauge specified instead of diam}

25   0 81.35 35       182.7 0 35   #12

25   182.7 0 35       0 -81.35 35  #12

25   0 -81.35 35      -182.7 0 35  #12

1 source

100,200,0

1 load

Resistor      {For impedance loads use anything other than "Laplace Transform"}

50,740,0      {Pulse # of the load, load resistance, load reactance}

Laplace Transform loads permit complex networks to be modeled, such as

tuned circuits.  A Laplace Transform is a special polynomial representation of

a lumped circuit.  Here is an example of an antenna using two Laplace Transform

loads:

W3DZZ Trap Dipole for 80 through 10 meters

Free space

14.150 MHz

4 wires, feet

10  0 -54 0   0 -32 0  #14

10  0 -32 0   0   0 0  #14

10  0   0 0   0  32 0  #14

10  0  32 0   0  54 0  #14

1 source

20,100,0

2 loads

Laplace Transform

10,2		{Pulse # of load, order of Laplace Transform}

0,1		{Numerator, denominator coefficents of s^0}

8.2,0		{    "           "          "       "  s^1}

0,4.92E-4	{    "           "          "       "  s^2}

30,2		{Second load ... }

0,1

8.2,0

0,4.92E-4	{Scientific notation is OK.  This is .000492}

The traps for this antenna consist of 8.2uH in parallel with 60 pF.

The Laplace Transform for a parallel LC circuit is Ls/(1+LCs^2).  L should be

given in uH and C in uF, since the program works in MHz internally.  Textbooks

on circuit theory contain explanations of Laplace Transforms.

Ground

For antennas modeled over ground the program will ask a series of

questions to establish the ground characteristics to be used for forming the

far-field directive pattern.  You can specify up to ten concentric ground zones

centered at the origin, each having its own dielectric constant, conductivity,

radius, and height.  This is very useful for modeling antennas on hilltops, in

swamps or marshes, or mounted on vessels above the water line.  The last ground

zone always extends to infinity.  When 0 is entered for the number of ground

zones the antenna will be modeled over a perfectly-conducting ground plane

extending to infinity.

For antennas over real earth a radial ground screen may be specified.

If you specify a ground screen and one ground zone, the soil characteristics

are assumed to be uniform.  If you specify a ground screen and two or more

zones, the soil under the radials is zone 1, with zone 2 beginning where the

radials end.  This is useful when the soil under the radials has been

chemically treated to increase its conductivity, and thus exhibits different

characteristics than the earth beyond.  The effective impedance of the ground

screen is combined in parallel with that of the earth underneath it when

determining the ground reflection factor used to calculate the far-field

directive patterns.  The ground screen model is most accurate for dense ground

screens employing more than 100 radial wires, but you should still use it for

smaller screens in order to obtain the best overall accuracy.

The height of the first ground zone is always 0.  The height of other

ground zones can be used to approximate elevation variations in local terrain.

If the height drops from one zone to the next, any diffraction from the cliff

edge thus formed is not modeled.  If the height increases, any blockage of

radiation from the wall formed is not modeled.  These are minor effects.

The antenna need not be located at the center of the ground zones.  The

ground zones are concentric with the origin (X=Y=Z=0), but you may place the

antenna at any coordinates.  You might use this to model reflection from a

small lake near the antenna, for example, or to more accurately model a very

long longwire or Beverage antenna which traverses several types of ground.

Caution needs to be exercised when modeling antennas over real earth.

MN uses the ground characteristics, including those of the ground screen, only

in determining the ground reflection factor for the far-field directive

patterns.  Perfectly-conducting earth is assumed when the element currents are

calculated.  This implies that neither direct nor induced ground current losses

will be accounted for.  For horizontal antennas at reasonable heights these

losses are negligible, but for vertical antennas fed against poor ground

systems they are not.  In the latter case the antenna gain calculated will be

too high and the impedance too low, and these numbers should be regarded as

upper and lower limits, respectively.  A resistive load may be inserted at the

feedpoint to approximate ground losses.

The most suitable reference for an antenna modeled over real ground is

not a dipole in free space, but a dipole or monopole substituted for the test

antenna with all other parameters unchanged.  Earth reflection coefficients

vary greatly with polarization and frequency, as well as with dielectric

constant and conductivity, so the reference antenna should operate at the same

frequency, with the same polarization, and with the same ground model as the

antenna being investigated.  Expect to see negative gains at very low elevation

angles, using realistic earth characteristics, for antennas that exhibit gain

in free space.  In particular, vertical antennas have very little response at

elevation angles near 0 degrees unless they radiate over salt water.  The same

is true for horizontal antennas over all types of ground, unless they are very

high.

Keep in mind that dBd refers to a dipole in free space, not to a dipole

in the same environment as the antenna being analyzed.  dBi is a somewhat more

natural reference than dBd when evaluating antennas over ground.

It takes 2-3 times longer to analyze an antenna over ground as in free

space.  When optimizing an antenna it is quicker to do as many preliminary runs

in free space as possible, leaving the final optimization over ground for last.

Pattern Generation

Azimuth and elevation patterns are generated to create the plot files

used by PLOT.  MN also searches these patterns to find the beamwidth and

maximum sidelobe level.  Normally, the azimuth and elevation patterns are only

generated for 0 to 180 degrees.  The pattern in the 0 to -180 degree half-plane

is assumed to be the same.  For antennas which do not possess mirror symmetry,

you can cause 360 degree patterns to be generated by using the parameter "360"

with the G command.  This feature can be used to find the true radiation

pattern of an asymmetrical wire antenna erected to take advantage of randomly

positioned tall trees, for example.  If the antenna is modeled over ground the

elevation data is always restricted to the half-plane above ground.

For free space models an elevation angle of 0 degrees is used during

the azimuth search to obtain the on-axis azimuth response.  For antennas over

ground you will be prompted for the elevation angle.  This permits you to focus

on a wave angle of interest for a particular propagation path.

The patterns are normally generated in 2 degree increments.  For some

antennas a smaller step size can produce better plots by resolving very sharp

peaks and nulls.  You can force the patterns to be generated in 1 degree steps

by adding the parameter "1" to the G command.  This can be used in combination

with the "360" parameter because MN just looks for "1" or "360" anywhere on the

G command line.  The time required to generate and search the patterns is

doubled when using 1 degree resolution or when generating 360 degree data, and

it is quadrupled when doing both.

If an azimuth or elevation search finds its maximum gain at 0 degrees

(the normal case for unidirectional antennas aimed in the +X direction) then

the 3 dB beamwidth is computed.  The beamwidth is found by locating the angle

on the positive angular side where the pattern drops more than 3 dB,

interpolating with the previous point, and then doubling the resulting value.

The interpolation enables the beamwidth to be accurately computed (for

symmetrical main lobes) even when searching data with 2 degree resolution.  The

other side of the main lobe is not examined, even if a 360 degree pattern is

generated.

If a sidelobe is found which is larger than the rear lobe then its

level and angle are displayed.  This is useful when optimizing the pattern of

an antenna.  It is easy to get carried away and lose sight of the overall

pattern when optimizing F/B, a measurement which describes the pattern at only

a single point in the rear plane.  You might achieve 45 dB F/B but still have a

sidelobe at 120 degrees that is only 20 dB down.  The sidelobe display will

alert you to this.  Only the first 180 degrees in each plane is searched for

sidelobes.

If the main lobe is found at an azimuth angle other than 0 then no

azimuth beamwidth or sidelobe information is given.  Instead, the gain and

azimuth angle of the main lobe are displayed, along with the level 180 degrees

to the rear, and the subsequent elevation search takes place in this plane

instead of at 0 degrees azimuth.  This is useful for longwires and other

antennas with skewed radiation patterns.

The radiation pattern used in all cases is the total pattern, which is

the RMS sum of the horizontal and vertical components.  This is realistic for

evaluating HF antenna performance on randomly polarized incoming skywave

signals.  It also facilitates the analysis of imperfect antennas.  For example,

cubical quads typically show only 20-30 dB front-to-side ratio due to

incidental vertically polarized radiation from the mostly out-of-phase sides of

the quad loops.  As another example, the performance of Beverage antennas can

be accurately modeled.  This antenna consists of a long low horizontal wire

which responds principally to vertically polarized signals.  The unintended

pickup of horizontally polarized fields will reduce directivity, but MN will

account for this.  Note that for linearly polarized antennas the magnitude

computed for orthogonal polarization terms is typically down more than 130 dB

in MN analysis, so that the use of both fields will not lead to inaccurate

results when the response at only one polarization is of interest.

SWR

MN computes SWR in two ways.  The first computation finds the SWR in a

50 ohm feedline attached to the antenna feedpoint.  This SWR value is displayed

every analysis run and is useful for antennas intended for direct feed.  (See

the "DOS Environment Variables" section below to change the 50 ohm value.)  The

second SWR computation uses the antenna input impedance at one frequency,

rather than the feedline impedance, as the SWR reference impedance.  This SWR

computation is designed to reveal the inherent bandwidth characteristics of the

antenna.

The second SWR value is displayed whenever you use the F command to

change frequency.  This value is based on a perfect match having been made to

the antenna at the first frequency analyzed after reading the antenna file.

The matching network is modeled as a series coil or capacitor followed by a

broadband transformer.  The coil or capacitor tunes out any residual reactance

in the antenna input impedance, and the transformer matches the resulting pure

resistance to the feedline impedance.

The variation in this second SWR will give you an idea of the inherent

bandwidth properties of the antenna, independent of any particular narrowband

matching network.  The only frequency-dependent aspect of the modeled matching

network is the small change in coil or capacitor reactance with frequency.  (If

you run the first analysis at the antenna resonant frequency this small effect

is eliminated altogether.)

If the antenna has multiple sources the SWR is computed at the first

source only.

Aborting Calculations

Because some of the calculations can take a long time (particularly if

a math coprocessor chip is not used), a provision has been made for aborting

from various program loops by pressing the <Esc> key.  The order of calculation

is such that the most fundamental results are available first.  This allows you

to use the abort feature to terminate unwanted calculations, as well as to

escape from command mistakes.  For example, if you are only interested in

obtaining input impedance, SWR, gain, and F/B, you may hit <Esc> after these

are displayed to terminate the azimuth and elevation pattern generation.  The

pattern data is used to find the beamwidth and maximum sidelobe levels, and

also for any subsequent plotting.  If you abort but later reanalyze, without

changing any parameters, the program will display the results previously

calculated, and immediately return to the pattern generation.  This avoids

duplicating the calculations already performed.

If it takes more than one minute to fill and factor the mutual

impedance matrix the program will beep when it is done to alert you that

results are ready.

DOS Environment Space

DOS provides a convenient way for you to specify configuration

information to MN and PLOT.  The DOS SET command places information into the

DOS Environment Space in memory, where it can be retrieved later by a program.

SET commands can be put into your AUTOEXEC.BAT file and they will be

automatically executed every time you boot the computer.

You may see what is currently in the DOS Environment Space by typing

SET.  You may eliminate an individual parameter by typing SET [parameter]=.

There are several SET parameters used by MN and PLOT:

1.  MN Editor

Your favorite text editor or word processor may be invoked from within

MN to edit the current antenna file.  To set this up do the following:

SET EDITOR=Editorname

where Editorname is the name of your editor (no .EXE or .COM needed).  Whenever

the E command is given to MN, the current antenna filename will be appended to

Editorname and the resulting DOS command will be executed.  You may specify a

drive and path ahead of Editorname.  After you exit the editor MN will reread

the antenna file.  All antenna parameters will be reset to those in the newly-

edited file, so any temporary changes made using the Change commands will be

lost.  The .RUN file is also overwritten each time the antenna file is read

back in.

2. Library Directories

When you accumulate many antenna and plot files it is nice to have

separate directories for them, so that the current directory doesn't get so

cluttered up.  You might use the current directory for antenna experiments,

saving optimized antenna files and their plots elsewhere.  You can tell MN and

PLOT to automatically reference another directory with SET commands.  You may

specify a directory for an antenna library and another for a plot library.

These libraries are used only for loading files; when MN creates a plot file

(or a .RUN file) it always writes it to the current directory.

If your antenna library is in the subdirectory ANTENNAS and your plot

library is in PLOTS then do the following:

SET ANTLIB=ANTENNAS

SET PLTLIB=PLOTS

You may specify a drive and path for the directories if necessary.

When this library facility is used MN and PLOT will list available files both

in the current directory and in the appropriate library directory.  To specify

a file from the library directory you only need to enter the filename.

MN and PLOT always search the current directory first.  If you need

to force the program to use the library (when the same filename appears in the

current directory) just specify the complete path and filename.

If you are using a dual floppy disk system you can put your antenna

library and plot library on a second disk drive.  This will give you plenty of

working space on your main drive and still allow access to the library files.

3. Reference dB

To cause MN and PLOT to display gain figures in dBi rather than dBd

do the following:

SET DB=dBi

You may use any combination of upper or lower case for the three dBi

letters.  Gains in dBd are referenced to the peak gain of a half-wave dipole in

free space.  This is common for amateur antennas and is the MN default.  Gains

in dBi are referenced to an isotropic antenna in free space, which is an

antenna that radiates equally in all directions.  A dipole has 2.15 dB gain

over an isotropic antenna, so MN converts from one gain reference to the other

by adding or subtracting 2.15 dB.

4.  Feedline Impedance

The reference feedline impedance for the SWR computation can be changed

from the default 50 ohms by doing the following:

SET Z=Feedline impedance

5.  Location of COMMAND.COM

MN needs to have access to the DOS command interpreter COMMAND.COM in

order to list antenna files, call your editor, and call PLOT.  PLOT needs

COMMAND.COM to list plot files.  Normally the DOS Environment Space will

contain the location of COMMAND.COM.  If it doesn't then you should do the

following:

SET COMSPEC=[Drive:\]COMMAND.COM

where Drive indicates the disk drive whose root directory contains COMMAND.COM

while MN is executing.  If you are using a single floppy disk system you must

copy COMMAND.COM from your DOS system disk onto the MN disk, or use a RAMDISK.

Summary of SET Commands

SET EDITOR=Editorname

SET ANTLIB=Antenna library directory

SET PLTLIB=Plot library directory

SET DB=dBi

SET Z=Feedline impedance

SET COMSPEC=[Drive:\]COMMAND.COM

Summary of MN Command Parameters

MN Command	Parameter			Function

----------      ---------       		--------

G		   1		Force 1 degree resolution for azimuth and

elevation patterns

G		  360		Force 360 degree coverage for azimuth and

elevation patterns

A		Filename	Specify a new antenna file without waiting

for the listing of available files

P		Filename	Create a plot file with a root name

different from that of the antenna file

Additional Information

NOSC Technical Document 938 contains a detailed description of the

original MININEC program, including mathematical formulation of the analysis

algorithm, model validation against empirical data, error analysis, and BASIC

program listing.  This 100+ page manual is available from the U.S. government

for $22.95.  It may be ordered by mail with a check, or by phone with a credit

card.  Request NTIS document number ADA181682 from:

U.S. Dept. of Commerce

National Technical Information Service

5285 Port Royal Rd.

Springfield, VA  22161

(703) 487-4650