| Ubiquiti Networks XtremeRange5 802.11a 600mW High Power Atheros Wireless mini-pci card Radio Specifications |  |
| --- | --- |
| Chipset | Atheros, 6th Generation, AR5414 with SuperA/Turbo Support |
| Frequency Range | IEEE 802.11a 5.2-5.825Ghz |
| Modulation | OFDM |
| Operating Sub-Channels | 13 USA ¦ 58 World non overlapping |
| Radio Output Power (at connector) | 600mW (28dBm) 1-24 Mbps +/-1.5dB 400mW (26dBm) 36 Mbps +/-1.5dB 316mW (25dbm) 48 Mbps +/-1.5dB 251mW (24dBm) 54 Mbps +/-1.5dB |
| Sensitivity @FER=0.08 | 6 Mbps -94 dBm +/-1.5dB 9 Mbps -93 dBm +/-1.5dB 12 Mbps -91 dBm +/-1.5dB 18 Mbps -90 dBm +/-1.5dB 24 Mbps -86 dBm +/-1.5dB 36 Mbps -83 dBm +/-1.5dB 48 Mbps -77 dBm +/-1.5dB 54 Mbps -74 dBm +/-1.5dB |
| Radio Data Rate | 54,48,36,24,18,12,6 with Auto Fall-Back |
| TX Channel Width Support | 5MHz / 10MHz / 20MHz / 40MHz |
| FCC Grant | FCC Part 15.247 (32dBi Antenna), IC RS210, CE with Notified Body Number |
| Power Consumption: If you are going to use the radios in a board that can not supply the necessary watts you must hard code the radio output power to what the board can handle. | Current Consumption at 3.3V 1-24Mb Transmit 1800mA, +/-100mA 36Mb Transmit 1500mA, +/-100mA 48Mb Transmit 1300mA, +/-100mA 54Mb Transmit 1100mA, +/-100mA 1-54Mb Receive 300mA, +/-100mA |
| Antenna Connector | 1x MMCX |
| Compatibility | Fully IEEE 802.11a compliant |
| Temperature Range | Storage -70C - +85C Operation -45C - +85C |
| Form Factor | 32-bit mini-PCI Type IIIA |
| Dimensions | 2 5/8"x 2" |
| Weight | 25g |
| RoHS Compliance | YES |
| Ubiquiti Networks XtremeRange5 802.11a 600mW High Power Atheros Wireless mini-pci card Networking Specifications |  |
| Network Architecture | Supports ad-hoc, peer-to-peer networks and infrastructure communications to wired Ethernet networks via Access Point |
| Access Protocol | CSMA/CA |
| Roaming | IEEE 802.11a compliant |
| Security | WPA, WPA2, AES-CCM & TKIP Encryption, 802.1x, 64/128/152bit WEP |